ES1879/1869/1868/1878/1888/1887

# AudioDrive®

# for Windows NT 4.0

# Version 2.00.82

# Software Release Notes

Aug 21,1997

# Contents

Contents	1

About This Release	2

Release Information	2

Release Description	2

KNOWN PROBLEMS	2

# About This Release

ESS Technology is providing a new Windows NT® driver for the ES1879/1869/1868/1878/1888/1887 AudioDrive® audio chip product in this release.

# Release Information

Product:	ES1879/1869/1868/1878/1888/1887 AudioDrive® for Windows NT® 4.0 Version 	2.00.82

Drivers:	ESS reference design of ES1879/1869/1868/1878/1888/1887

OS:	Microsoft Windows NT® 4.0 or later

Release Date:	Aug 21,1997

# Release Description

New Changes in Version 2.00.82

- Support Single Transfer DMA mode from line “SingleTransferDMA=1” in file “config.ini”.

- Support Single DMA (half-duplex) for ES1869/ES1879.

- Added Mono-In mixer line into ES1869.

Changes in Version 2.00.79

- Greyed out the “setting” box in the multimedia devices properties dialogbox after installation is done automatically. This is to prevent the driver from being disabled from    user by clicking it.

- Support    ES1887 automatic installation if the chip is enabled by BIOS PnP.

Changes in Version 2.00.74.

- Added    ES1879 support.

- Provide 16 bit DMA support for either 1st or 2nd channel for ES1879/1869/1868/1878.

Changes in Version 2.00.73.

- Combined automatic installation and standard installation. If ESS audio chip was already configured by BIOS,    the resource settings(irq, dma, base port, etc) will be read from the chip and installation will not ask to the user. But if it is not successful, the installation procedure will ask the user for those resource information from dialog box.    This feature can be disabled by adding “EnableAutoInstall=0” into file “config.ini” so that installation always forces to standard installation.

Changes in Version 2.00.70.

- Add support for ES1869.

- Better frequency response and more accurate sample rate algorithm was implemented.

- Give the flexibility of choosing full-duplex or half-duplex depending on how many DMA channles are available.

- Changed the driver file name from “essaudio.dll” to “auddrive.dll”.

- Changed default PnP configuration port base address from 620 to 800 because the most reference designs use 800.

- Fixed 1869 spatializer default as “on”.

# Known Problems

When we try to change MIDI device port from ESFM to external MIDI or vice versa (for example, from MIDI properties of media player), it does not take an effect immediately.    Rather, it takes an effect only after rebooting the system. It seems to be a    Microsoft NT 4.0 bug.